CMPE 311

Fall 2026

Project #2: Bare Metal Asynchronous Tasking

Colin Cole

# Part #1

PROBLEM TO BE SOLVED: From The Customer’s requirements in Table 1, derive the high-level technical requirements for the system being designed. These requirements are written in the Customer’s language and understandable by the Customer. Typically, these requirements are not specific enough to be used for engineering purposes. For example, the customer requirement might state that the resultant device must be designed for an “averaged size man”. In contrast, technical requirements include specific parameters that can be designed to, e.g. “someone greater than 165 cm and less than 191 cm.” Table 2 provides examples of some high-level (HL) technical requirements

### Customer:

An annoying Professor.

### Customer Requirements

C1. The User will be able to choose the blink rate of the LED with a button.

C2. The User will choose between 4 rates plus off.

C3. The blink rates must be set by an operator prior to the system being started.

C4. Permitted blink rates must be perceivable by the viewer.

C5. The minimum blink rate must be no slower than once every 5 seconds.

C6. The System must run upon an Arduino Uno R3 compatible board.

### High-Level Technical Requirements

HL1. The developer must use an Arduino or Arduino compatible development board.

HL2. The blink rate of the LED must be between once every 100ms and once every 5 seconds.

HL3. The system must have five states corresponding to four blink rates and the off state.

HL4. The system must use one button and one LED.

HL5. The system must communicate with the user via serial.

HL6. Each press of the button should advance the state of the LED by one, such that it toggles through all 4 blink states and the off state one at a time.

HL7. Each blink cycle should have a 50% duty cycle.

HL8. The system should allow an operator to choose the blink rates via the Arduino code before the system starts.

# Part #2

PROBLEM TO BE SOLVED: Given the high-level technical requirements (HL.#) and educational constraints (EC.#) below – and different from the one’s you derived in Part II – derive the testing requirements (labeled T.#) that will allow The Company to prove to The Customer that The Company has satisfied The Contract.

### High-level technical requirements

HL.1 The System must use at least 2 LEDs

HL.2 The System must use a standard Arduino compatible development board (e.g. the provided ELEGOO Uno R3)

HL.3 The System must communicate with The User via the Arduino IDE serial-monitor port

HL.4 Any use of the serial-monitor in HL.3 must be asynchronous and not affect the blinking of the LEDs.

HL.5 The User must set the blink interval of the LEDs in msec

HL.6 The blink rate of each LED must be constant unless changed by The User

### Educational Constraints

EC.1 The Developer must use the version of gcc used by Arduino IDE 2.3.3 or later for the AVR ATmega328p to cross compile any program used by The System

EC.2 The Developer must have any software used in The System hosted on a github repository. This software does not need to include libraries, macros or other code not developed by The Developer.

EC.2.1 The Evaluator must be able to (a) download from the repository any software, documenta on and other material needed for their evaluation, (b) cross compile any software, and (c) run that software on The Reference Platform.

EC.3 LEDs must be connected to Arduino digital Pins 2 & 3.

EC.4 The Evaluator must be able to successfully have The System perform the dialog in Table 5.

EC.5 The Developer must do bare metal programming, i.e. they cannot use any 3rd party multi-tasking libraries or programs such as FreeRTOS.

### Testing Requirements

T.1 Inspect the assembly and verify its operation. There should be at least 2 LEDs present, and they should be able to blink.

T.2 Inspect the board. It should be an Arduino compatible development board that communicates with a PC through serial and Arduino IDE.

T.3 The system must allow the user to set the blink rates of the LEDs in ms, which then should be measured with an oscilloscope to verify the correct timing.

T.4 Configure the LEDs with different intervals and monitor the LEDs to make sure that they maintain their configured intervals.

T.5 Change one LED while both are blinking. The unchanged LED should maintain the configured blink rate.

T.6 Inspect the build configuration of the program and ensure that it is using an allowed version of gcc and Arduino IDE.

T.7 Access the GitHub repository with the permissions intended by the developer and insure all required code is visible and accessible.

T.8 From a clean evaluation environment, download the developer’s code from GitHub and ensure that it runs with no changes or additional libraries.

T.9 Inspect the wiring of the board and ensure that the LEDs are wired separately and connected to ports 2 and 3.

T.10 The implementation satisfies the stated bare-metal constraint and contains no third-party multitasking library or program, including FreeRTOS.

T.11 The implementation must utilize a round-robin cyclic executive task manager.

# Part #3

Given the high-level (HL.#) and educational constraints (EC.#) above, see Tables 3 & 4, as well as the testing requirements you derived in Part II (T.#), design and build The System (i.e. the device). Turn in a design document which will include any schema cs, rational and assumptions required for the reader to understand your design. Use the testing requirements you derived in Part II to evaluate The System. Include the results (including data, pictures and videos) of your tests (T.#) that validate your device (i.e. prove that your device works as it should). NOTE: The design document is not a running commentary of your development process! It is only the material that a competent engineer requires to reproduce the device. (E.g. the following statement is unneeded to recreate the device, “A er we tried design#1, we decided it was too big and went on to design#2, which did work”.)

### Required Equipment

- ELEGOO Uno R3

- LED(2x)

- 200Ω ¼ Wa resistor(2x)

- 10kΩ ¼ Wa resistor

- Arduino IDE 2.3.3 or better

- Laptop or other computer to run the Arduino IDE

- Cable, USB 2.0 female type B to female USB 2.0 type A

- Breadboard

- Varied length wires

### Physical Design Photo

### Arduino Code & Explanation

#define LED_1_STATE (1 << 0) #define LED_2_STATE (1 << 1) #define WAITING_PIN (1 << 2) #define WAITING_INT (1 << 3) #define INPUT_PIN (1 << 4) uint8_t flags = 0; char inputBuffer; unsigned long currentTime = 0; struct LED{ unsigned long blinkIntervalMS = 0; unsigned long blinkStartTime = 0; }; LED led1; LED led2; void setup() { Serial.begin(115200); delay(1000); pinMode(2, OUTPUT); pinMode(3, OUTPUT); led1.blinkIntervalMS = 1000; led2.blinkIntervalMS = 1000; digitalWrite(2, HIGH); digitalWrite(3, HIGH); flags ¦= LED_1_STATE ¦ LED_2_STATE; currentTime = millis(); led1.blinkStartTime = currentTime; led2.blinkStartTime = currentTime; } void loop() { currentTime = millis(); if(flags & LED_1_STATE){ if(currentTime-led1.blinkStartTime > led1.blinkIntervalMS/2){ digitalWrite(2,LOW); flags &= ~LED_1_STATE; led1.blinkStartTime = currentTime; } } else{ if(currentTime-led1.blinkStartTime > led1.blinkIntervalMS/2){ digitalWrite(2,HIGH); flags ¦= LED_1_STATE; led1.blinkStartTime = currentTime; } } if(flags & LED_2_STATE){ if(currentTime-led2.blinkStartTime > led2.blinkIntervalMS/2){ digitalWrite(3,LOW); flags &= ~LED_2_STATE; led2.blinkStartTime = currentTime; } } else{ if(currentTime-led2.blinkStartTime > led2.blinkIntervalMS/2){ digitalWrite(3,HIGH); flags ¦= LED_2_STATE; led2.blinkStartTime = currentTime; } } //User Input if (flags & WAITING_PIN) { if (Serial.available() > 0) { inputBuffer = Serial.read(); switch (inputBuffer) { case '\r': case '\n': break; case '1': case '2': if (inputBuffer == '2') { flags ¦= INPUT_PIN; } else { flags &= ~INPUT_PIN; } flags &= ~WAITING_PIN; flags ¦= WAITING_INT; Serial.println("Enter an interval from 1 to 60000 ms:"); break; default: if ((unsigned char)inputBuffer >= 32 && (unsigned char)inputBuffer <= 126) { Serial.println("Invalid LED. Enter 1 or 2:"); } break; } } } else if (flags & WAITING_INT) { static unsigned long value = 0; static bool hasDigits = false; static bool invalid = false; while (Serial.available() > 0) { char c = Serial.read(); if (c == '\r') continue; if (c == '\n') { if (!hasDigits && !invalid) continue; bool valid = hasDigits && !invalid && value > 0; if (valid) { if(flags & INPUT_PIN){ led2.blinkIntervalMS = value; flags &= ~WAITING_INT; } else{ led1.blinkIntervalMS = value; flags &= ~WAITING_INT; } } else { Serial.println("Enter an interval from 1 to 60000 ms:"); } value = 0; hasDigits = false; invalid = false; if (valid) break; } else if (c >= '0' && c <= '9') { hasDigits = true; if (!invalid) { value = value * 10 + (c - '0'); if (value > 60000) invalid = true; } } else { invalid = true; } } } else{ Serial.println("What LED? (1 or 2)"); flags ¦= WAITING_PIN; } }

#ifndef MY_MILLI_H #define MY_MILLI_H #include <stdint.h> // Custom defined data memory adresses #define MY_TIMER1_CONTROL_A (*(volatile uint8_t *)0x80) #define MY_TIMER1_CONTROL_B (*(volatile uint8_t *)0x81) #define MY_TIMER1_COUNT_HIGH (*(volatile uint8_t *)0x85) #define MY_TIMER1_COUNT_LOW (*(volatile uint8_t *)0x84) #define MY_TIMER1_COMPARE_HIGH (*(volatile uint8_t *)0x89) #define MY_TIMER1_COMPARE_LOW (*(volatile uint8_t *)0x88) #define MY_TIMER1_INTERRUPT_MASK (*(volatile uint8_t *)0x6F) #define MY_TIMER1_FLAGS (*(volatile uint8_t *)0x36) #define MY_CPU_STATUS (*(volatile uint8_t *)0x5F) // bit masks and timer constants #define MY_TIMER1_CTC_MODE (1u << 3) #define MY_TIMER1_DIVIDE_BY_64 ((1u << 1) ¦ (1u << 0)) #define MY_TIMER1_COMPARE_A_INTERRUPT (1u << 1) #define MY_TIMER1_COMPARE_A_FLAG (1u << 1) #define MY_TIMER1_COMPARE_VALUE 249u // AVR startup vector table #define MY_TIMER1_COMPARE_HANDLER __vector_11 static volatile uint32_t myMilliCount = 0; // replace the supplied ISR() helper macro. extern "C" void MY_TIMER1_COMPARE_HANDLER(void) __attribute__((signal, used, externally_visible)); extern "C" void MY_TIMER1_COMPARE_HANDLER(void) { ++myMilliCount; } static inline void myMilliBegin(void) { const uint8_t savedStatus = MY_CPU_STATUS; // A CPU instruction, not the AVR cli() helper macro. __asm__ __volatile__("cli" ::: "memory"); MY_TIMER1_INTERRUPT_MASK = 0; MY_TIMER1_CONTROL_B = 0; // Stop the timer before changing it MY_TIMER1_CONTROL_A = 0; // Clear remaining mode/output settings // Timer1's 16-bit registers require high-byte-first writes MY_TIMER1_COUNT_HIGH = 0; MY_TIMER1_COUNT_LOW = 0; MY_TIMER1_COMPARE_HIGH = 0; MY_TIMER1_COMPARE_LOW = MY_TIMER1_COMPARE_VALUE; myMilliCount = 0; // This flag is cleared by writing a ONE, not a zero MY_TIMER1_FLAGS = MY_TIMER1_COMPARE_A_FLAG; MY_TIMER1_INTERRUPT_MASK = MY_TIMER1_COMPARE_A_INTERRUPT; // 16,000,000 / 64 / (249 + 1) = 1,000 interrupts per second MY_TIMER1_CONTROL_B = MY_TIMER1_CTC_MODE ¦ MY_TIMER1_DIVIDE_BY_64; // Preserve the caller's global interrupt state MY_CPU_STATUS = savedStatus; __asm__ __volatile__("" ::: "memory"); } static inline uint32_t myMilli(void) { const uint8_t savedStatus = MY_CPU_STATUS; __asm__ __volatile__("cli" ::: "memory"); // Prevent an ISR from changing it partway through this read. const uint32_t snapshot = myMilliCount; MY_CPU_STATUS = savedStatus; __asm__ __volatile__("" ::: "memory"); return snapshot; } #endif

The program starts by including Timer.h and defining macros. These macros allow the program to utilize a single uint8_t as a bit field to store various flags, rather than defining multiple separate variables. This saves space, as a bool still takes up a whole byte. It then defines a struct for the LED to make reading easier and to store the LED-specific variables in a container. Timer.h provides the custom timer, using Timer1 to trigger an interrupt every millisecond. The interrupt handler increments a counter, which myMilli() safely reads. The setup code then runs, initializing the serial connection, adding a short delay, and initializing the LED pins and variables. It calls myMilliBegin() to start the timer and records the LEDs’ starting times. The main loop then runs, sampling myMilli() into currentTime before calling each task. First, it checks the LEDs to see if they are due for a state change. After that, it handles the asynchronous user input. Serial.available() checks for input before Serial.read() reads it, allowing the program to continue without waiting. The first if block waits for the user to select an LED. After selection, it updates the flags to match the current input state. When the user enters an interval, it reads through the serial buffer, validates the value, sets the corresponding blink rate, and resets the input state for the next selection.

### Video Link

# Engineering Change Note 1

ECN Number: 1

Date: 9/29/2026

Author: Colin Cole

## Purpose and Reason for Change

The project has been amended to use a polled, round-robin executive task manager based on a function pointer table, as required by ECO 0.1. This change organizes tasks and supports future additions to the system.

## Customer requirements

The customer requirements remain unchanged.

## High-level technical requirements

The high-level technical requirements remain unchanged from 1, except for the addition of a requirement to use a round-robin Cyclic Executive task manager. The intended LED and serial-interface behavior is preserved.

## Engineering changes

The PROJECT-CE software uses a function-pointer array named tasks[] containing:

- taskLED1(): checks and updates LED 1.

- taskLED2(): checks and updates LED 2.

- taskSerial(): processes LED selection and blink-interval input.

During each execution of loop(), the software samples millis() and calls every task in array order. Each LED task checks its elapsed time to determine whether to toggle its output. The serial task checks for available input and maintains its input state between calls.

## Updated Video Link

https://drive.google.com/file/d/1DaqNN1ieI-Ej2T9UFDhZFlNygwyh_VbI/view?usp=sharing

# Engineering Change Note 2

ECN Number: 2

Date: 10/15/2026

Author: Colin Cole

## Purpose and Reason for Change

The project has been amended to use a custom defined timer instead of the provided millis() function.

## Customer requirements

The customer requirements remain unchanged.

## High-level technical requirements

The high-level technical requirements remain unchanged from 2, except for the addition of a requirement to use a custom myMillis() implimentation

## Engineering changes

The PROJECT-CE software replaces Arduino’s millis() function with custom timekeeping defined in Timer.h, containing:

- myMilliBegin(): configures Timer1 to generate an interrupt every millisecond.

- Interrupt handler: increments a millisecond counter after each timer compare match.

- myMilli(): safely reads and returns the counter.

- Timer1 uses CTC mode, a prescaler of 64, and a compare value of 249 for a 16 MHz processor. Register addresses, bit masks, and the interrupt handler are defined without the provided AVR timer macros.

During setup(), the software initializes the timer and records each LED’s starting time. During each execution of loop(), it samples myMilli() into currentTime and calls every task in array order. The LED tasks use currentTime to determine when to toggle their outputs.